| Lee Sichello Thunder-Clap Electronics 1234 Any St. Suite #15 Townsville, SK S4S5B9 | Tel: 306-529-7777 Fax: 306-555-1234 thunder_clap@electronics.ca |
| --- | --- |

Prepared for: 	Eddy McDonald

45 Nowhere Street, Regina, SK S4S0H3

tel (306) 589 9876

eddy@mcdonald.ca

Prepared by:	Lee Sichello, Thunder-Clap Electronics

987 Electric Avenue, Regina, SK S4S5B9

tel (306) 555 1234

thunder_clap@electronics.ca

February 2nd, 2011

File Version: 1.00A

Proposal Number: 5454-1

Abstract

Ed McDonald requested that a 'smart power bar' be designed to plug into a wall socket. Power usage on multiple sockets is to be logged and displayed on an LCD screen. Users must be able to input power costs per kWh and power bar is to make a slection of cost calculations for each outlet.

Background

There are a variety of energy meters available on the market but consumers are very limited in their options. The vast majority include only one outlet and are incapable of performing the cost calculations that many consumers find challenging.

Proposal

Description

A power bar will be created to measure the power usage for two 110 VAC electrical outlets on the module. Measurements will need to be logged in memory for averaging and cost calculations. To allow for input of local power costs, there will be a series of pushbuttons for user control. Users will have an option to input local power costs as well as a series of easily selectable options for what is displayed on the LCD screen. All displayed measurements and calculations must be within 10% error.

Overview and Operation

The user selectable options on the LCD screen for each device must include:

- Instantaneous current use

- Instantaneous power use

- Average energy use

- Total energy use

- Estimated cost per day of operation

Users will be able to page through these options at will.

Put simply, electrical power usage at any instant is the product of voltage and current. Since this device will be used with 110 V, 50Hz AC, only current will be measured. Any deviations from true 110 VAC residential power will be made up for by my 10% error margin.

Current sensing will be done via a series of current transformers (CT's). Current will be sampled, via and ADC, at a much higher frequency than the 50Hz supply power. By logging many samples each cycle, an RMS current calculation can be made and saved at regular time intervals (at least once a second), then samples may be cleared from memory. Each time RMS current is calculated, energy use will be calculated via:

E = Prms * Time Elapsed

Rather than keeping an indefinite running total of consumed energy, the unit will accumulate up to 0.01 kWh (the resolution of the screen) then increment.  By keeping track of the energy consumed and the cost per kWh, it is straight forward to calculate the cumulative cost or forecast the cost per day of an appliance.

The unit will be powered up and acquiring data whenever plugged into a wall socket. If unplugged, all former data will be cleared from memory. This can also be achieved by means of a master reset button. Users can also reset the data for individual channels using the pushbuttons.

Upon startup, the device will enter a default mode of operation and remain there until the user interrupts it via the inputs. Interrupts include selecting display options and entering the “PROGRAM” mode to input local power costs.

Operating Conditions and Safety

Being a high voltage device, safety is a major concern. For this reason, the unit will be electrically isolated from the 110 VAC with the exception of the step-down transformer used in the power supply. The current transformers will provide this isolation.

To protect from over-current, the main input will be equipped with either a fuse or circuit breaker. The unit must be able to operate between 0 and 50 degrees Celsius (domestic use) with humidity up to 80%.

Physical Packaging

The prototype will be housed completely within a 10x6x3.5” aluminum project box. The box will have a single power cord for 110 VAC power connection.

Budget

The budget for the initial working prototype is a fixed cost of $3000. The production power bars must be under $100 though. If it is not built by the deadline, a discount of 10% per month will be given until the project is finished.

Optional Additional Features

For safety considerations, surge protection may be added to cut power in such an event. Also, ground-fault interruption  could be implemented by measuring return current on the neutral main line. If this current is not equal to the sum of the currents in the devices, power should be cut and a signal LED triggered.

Time permitting, an on-board battery powered clock could be added to the unit. Then users could set the time and schedule times for devices to turn off/on throughout the day.

Bill Of Materials

- Master Control Unit

  - Microprocessor

    - Wytec Dragon12 module (selected for its second hand availability)

  - Analog Inputs

    - Push Buttons

    - Analog to Digital Conversion

      - HC9S12 A/D 8 or 10 bit Sigma Delta ADC

      - Input conditioning

        - LM358 Low Power, Single Supply, Dual Operational Amp

        - Resistors and capacitors as required to filter and scale input

  - Digital Outputs

    - Hitachi 44780 LCD Controller

  - Power Supply

    - Universal AC/DC Power Adaptor Supplies +5 to +7VDC power

- Current Sense

  - Current Transformers - (2) CT1015 , 1:1000, 15Amp

- Wiring and Connectors

  - Power Cable (110VAC) – 14 Guage, LIVE/NTRL/GND stranded wire’

  - 1 Male, 2 Female Standard 110VAC Plugs

  - Signal Jumpers – Connect Current Sense to MCU

- Safety

  - Circuit Breaker (8 Ohm)

Logical and Physical Layout

Logical Connection Diagram

Physical Layout Diagram

Input Conditioning

The output from the CT’s is an AC voltage proportional to the current being used. In order to interface this with the ADC inputs, it must be scaled from 0-5V. To do this, the CT voltage is half-wave rectified, then passed through a single supply (+5V) op-amp which acts as a protective buffer. The half-waves are then smoothed to a DC signal by capacitors. The circuit looks as follows:

LCD/Pushbutton Interface

When plugged in, the unit will boot to a default mode. In this mode, it will simply display the average power use for outlet 1, as indicated below.

To change between outlets (indicated by LED’s) simply hit the OUTLET button. In order to clear the data for the present outlet, hold down the OUTLET button for 4 seconds. To toggle display options for the present outlet use the LEFT and RIGHT arrows. In order to program in the cost per kWh you are being charged, press the PGRM button then use the LEFT and RIGHT arrows to set the value. Once the value is set, hit PGRM again to return to normal display, or wait for 10 seconds.

Software Design

The main program of the firmware will initialize the hardware and interrupts, then wait for interrupts to set a proceed flag to continue. The interrupt routines will be used to perform measurements, do calculations and set flags for use in the main program. The interrupts used will be:

- Real-Time Interrupts

  - Sample current 5 times per 60Hz AC cycle (that is, every 3 ms), log data.

  - Calculate instantaneous current & power use, and update total energy use every 60 AC cycles (every 1s)

  - Estimate average energy use and cost per day every 180 cycles (every 3s)

  - Check pushbutton status (every 1ms)

Flow Chart Of Main Program and Some RTI’s (more to come).

System Testing

Testing will be done using an in-line AC ammeter (to measure actual current flow) and a load with variable resistance. The current will be ramped up and the ammeter readings will be compared to the instantaneous current readings on the unit to ensure that the two are within a 10% agreement.

Detailed Schedule

- Complete Input Conditioning circuit – by Mar 12

  - Finish prototyping

  - Develop transfer function for current vs. voltage output

  - Solder together

- Assemble project in aluminum housing – by Mar 17

  - Wire in AC circuitry

  - Fix input conditioning board and Dragon 12 in place

- Push button interrupts – by Mar 20

  - Program proper reaction for each button

- Real-time interrupts – by Mar 25

  - Aquire current samples

  - Do power and cost calculations

- Troubleshoot and Complete Unit – by Apr 3

Completed Unit

Design vs Reality

I am extremely pleased with the end result. All of the deliverables layed out in the functional specification have been met by the ‘Smart Power Bar.’ The unit is capable of sampling and displaying the current for two outlets within a 10% error. The unit is capable of taking user input for the local cost per kilowatt-hour. Using the current sample and local cost, it can calculate and display the instantaneous power use, average power use, total energy use from startup, total cost incurred and the estimated cost for a full day of operation for each outlet independantly. Display and user input protocol are a match with the initial design.

The unit is packaged in an aluminum box which is smaller that the maximum specs mentioned. The case is grounded and utilizes a 15 Amp breaker for safety considerations. I changed from a 8Amp to 15 Amp breaker since the loads I wished to drive would’ve been popping the breaker.

Things which I did not mention in my design which have been included in the final product are that my 5V DC power supply is external to the aluminum case (so it can be reused) and there are holes bored in the left side of the case so that a serial connection can be made. Also, there are holes in the case for access to the Dragon12 Bootloaded switches and reset button.

Hours Spent on Project

Based on the assumption that I have spent 10 hours per week on the project since the beginning of reading week, plus 10 hours on initial design and another 10 on documentation; I estimate that I’ve spent around 80 to 90 hours working on my project. This is a lot of time but I think it has been reflected in the final result.

Testing Plan With Results

As mentioned, the testing plan was to be done using an in-line AC ammeter (to measure actual current flow) and a load with variable resistance. The current was ramped up and the ammeter readings were compared to the instantaneous current readings on the unit to ensure that the two are within a 10% agreement.  In reality, I was closed to a maximum error of about 2%, but it was nice to have that extra padding. Also, once the variance of the incoming 120V power was folded into the mix, the error may have been closer to 10%.

Conclusions/Lessons Learned

Again, I am quite pleased with my results for the project. It was time consuming and frustrating at times but also very rewarding in the end. The biggest lessons I learned, aside from the obvious hardware interfacing and assembly code, were:

- Always backup working versions of your code in case something goes awry and you can’t fix it.

- Break things up into small chunks and focus on them one at a time or it’s easy to get overwhelmed.

- The least important aspects and little details are often the most time consuming in a project like this.

Final Block Diagram

Detailed Schematic

Source Code:

Main

;Project:         Smart Power Bar

;Creator:         Lee Sichello

;Date:          Jan-Mar, 2011

;Class:         ENEL 387

;Description:   The following is the assembly code written to control the

;'smart power bar' module. IT is written for the HC9S12

;and implemented on a Dragon12 board from WYTEC along with a

;custom designed current sampling circuit. The program makes use

;of the LCD screen for output and uses the pushbuttons for user

;control. Two ADC channels are the only external interfaces.

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

#include        D_variant_registers.inc

REGBLK:         equ     $0000

STACK:          equ     $2000

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

org     $1000

counter:        rmb     1

pbtn_status:    rmb     1

pbtn_sample:    rmb     1

pbtn_count:     rmb     2

disp_no:        rmb     1

hold_count:     rmb     2

pgm_count:      rmb     2

intFLG:         rmb     1                     ;used to enter into main loop

cost:           rmb     2

daily_buffer:   rmb     7

cost_buffer:    rmb     7

v_buffer:       rmb     6

i_buffer:       rmb     6

p_buffer:       rmb     7

Pave_buffer:    rmb     7

totE_buffer:    rmb     7

totC_buffer:    rmb     7

n_nullsamp1:    rmb     2

sum_samp1:      rmb     2                     ;sum of 1ms voltage samples

n_samp1:        rmb     2                     ;number of 1ms samples

sum_ave1:       rmb     2                     ;sum of average reading for 60 cycles

n_ave1:         rmb     2                     ;number of 60 cycle averages

v_inst1:        rmb     2                     ;'instantaneous' voltage = sum_ave50cyc/n_ave50cyc(taken every 2000 samples or 2s)

i_inst1:        rmb     2                     ;instantneous current

p_inst1:        rmb     2

n_nullsamp2:    rmb     2

sum_samp2:      rmb     2                     ;sum of 1ms voltage samples

n_samp2:        rmb     2                     ;number of 1ms samples

sum_ave2:       rmb     2                     ;sum of average reading for 60 cycles

n_ave2:         rmb     2                     ;number of 60 cycle averages

v_inst2:        rmb     2                     ;'instantaneous' voltage = sum_ave50cyc/n_ave50cyc(taken every 2000 samples or 2s)

i_inst2:        rmb     2                     ;instantneous current

p_inst2:        rmb     2

Pave1:          rmb     2

Pave2:          rmb     2

Iave1:          rmb     2

n_Iave1:        rmb     2

Iave2:          rmb     2

n_Iave2:        rmb     2

daily1:         rmb     2

daily2:         rmb     2

tcount1:        rmb     2

tcount2:        rmb     2

seconds1:       rmb     2

seconds2:       rmb     2

totE1:          rmb     2

totE2:          rmb     2

totcost1:       rmb     2

totcost2:       rmb     2

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

org     $4000                        ;begin program code

handler_0:      rti

start:          lds     #STACK

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

#include        init_hrdwr.inc                  ;initialize the hardware

#include        init_intrpt.inc                 ;initialize the RTI

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

main:

movw    #$0000,v_inst1          ;initialize variables

movw    #$0000,v_inst2

movw    #$0000,i_inst1

movw    #$0000,i_inst2

movw    #$0000,p_inst1

movw    #$0000,p_inst2

movw    #$0000,n_nullsamp1

movw    #$0000,sum_ave1

movw    #$0000,n_samp1

movw    #$0000,sum_samp1

movw    #$0000,n_ave1

movw    #$0000,sum_ave1

movw    #$0000,n_nullsamp2

movw    #$0000,sum_ave2

movw    #$0000,n_samp2

movw    #$0000,sum_samp2

movw    #$0000,n_ave2

movw    #$0000,sum_ave2

movb    #$00,intFLG

movw    #$0000,cost

movb    #$00,pbtn_status

movb    #$01,disp_no

movw    #$0000,Iave1

movw    #$0000,n_Iave1

movw    #$0000,Iave2

movw    #$0000,n_Iave2

movw    #$0000,Pave1

movw    #$0000,Pave2

movw    #$0000,daily1

movw    #$0000,daily2

movw    #$0000,hold_count

movw    #$0000,tcount1

movw    #$0000,tcount2

movw    #$0000,seconds1

movw    #$0000,seconds2

movw    #$0000,totE1

movw    #$0000,totE2

movw    #$0000,totcost1

movw    #$0000,totcost2

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

back:

jsr     DISPLAY                         ;display proper output

check_pgm:

brclr   pbtn_status,#$01,check_out      ;program cost if needed

jsr     PROGRAM

check_out:

brclr   pbtn_status,#$02,check_done     ;change/reset outlet

jsr     OUTLET                          ;if needed

check_done:

jmp     back                            ;repeat

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

#include        CNTRL_FNCTNS.inc        ;functions for flow control

#inclde         PRINT_CMNDS.inc         ;various output commands

#include        BUFF_PREP.inc           ;ascii buffer preparation

#include        LCD_cmnds.inc           ;basic LCD commands

#include        RTI_ISR.inc             ;Real Time Interrupt

#include        DATA.inc                ;output strings

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

#include        vector_table.inc

End

;CNTRL_FNCTNS.inc

;Lee Sichello

;Purpose: A set of subroutines which alter the flow of control and user flags in the program. Makes output decisions

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

DISPLAY:

brset   pbtn_status,#$04,inc_display

brset   pbtn_status,#$08,dec_display

jmp     display_set

dec_display:    ldaa    #$01

cmpa    disp_no

beq     set_disp6

dec     disp_no

bra     display_set

set_disp6:

ldaa    #6

staa    disp_no

bra     display_set

inc_display:    ldaa    #$06

cmpa    disp_no

beq     set_disp1

inc     disp_no

bra     display_set

set_disp1:

ldaa    #1

staa    disp_no

bra     display_set

display_set:

movb    #$00,pbtn_status

jsr     clear_LCD

disp_1:         ldaa    #$01

cmpa    disp_no

bne     disp_2

jsr     print_i

rts

disp_2:         ldaa    #$02

cmpa    disp_no

bne     disp_3

jsr     print_p

rts

disp_3:         ldaa    #$03

cmpa    disp_no

bne     disp_4

jsr     printPave

rts

disp_4:         ldaa    #$04

cmpa    disp_no

bne     disp_5

jsr     print_totE

rts

disp_5:         ldaa    #$05

cmpa    disp_no

bne     disp_6

jsr     print_totC

rts

disp_6:         jsr     print_daily

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

PROGRAM:

bclr    pbtn_status,#$01

jsr     clear_LCD

ldx     #str_pgm

jsr     putsLCD

ldaa    #$C0

jsr     cmd2LCD

ldd     cost

ldy     #cost_buffer

jsr     prep_1dp_buff

ldx     #cost_buffer

jsr     putsLCD

ldx     #str_cents

jsr     putsLCD

movw    #$0000,pgm_count

bclr    intFLG,#$03

pgm_back:

brclr   intFLG,#$03,pgm_check_stat   ;check pbtns until time is up

bclr    intFLG,#$03                  ;clear timeout flag

rts

pgm_check_stat:

brset   pbtn_status,#%00001000,pgm_L ;go to left routine if pressed

brset   pbtn_status,#%00000100,pgm_R ;go to right routine if pressed

jmp     pgm_back

pgm_L:

ldd     cost

cpd     #0

beq     print_new_cost

subd    #5

std     cost

bclr    pbtn_status,#%00001000

bra     print_new_cost

pgm_R:

ldd     cost

cpd     #500

beq     print_new_cost

ldd     cost

addd    #5

std     cost

bclr    pbtn_status,#%00000100

jmp     print_new_cost

print_new_cost:

ldaa    #$C0

jsr     cmd2LCD

ldd     cost

ldy     #cost_buffer

jsr     prep_1dp_buff

ldx     #cost_buffer

jsr     putsLCD

ldx     #str_cents

jsr     putsLCD

movw    #$0000,pgm_count

bclr    intFLG,#$02

jmp     pgm_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

OUTLET:

movw    #$0000,hold_count

outlet_back:

brclr   pbtn_status,#$02,toggle_outlet

ldy     hold_count

cpy     #2000

bne     outlet_back

brset   intFLG,#08,reset2

movw    #$0000,Iave1

movw    #$0000,n_Iave1

movw    #$0000,Pave1

movw    #$0000,daily1

movw    #$0000,tcount1

movw    #$0000,seconds1

movw    #$0000,totE1

movw        #$0000,totcost1

bra     outpt_rst

reset2:         movw    #$0000,Iave2

movw    #$0000,n_Iave2

movw    #$0000,Pave2

movw    #$0000,daily2

movw    #$0000,tcount2

movw    #$0000,seconds2

movw    #$0000,totE2

movw    #$0000,totcost2

bra     outpt_rst

toggle_outlet:

brset   intFLG,#$08,clear_outflg

bset    intFLG,#$08                     ;toggles the outlet

rts

outpt_rst:

jsr     clear_LCD

ldx     #str_rst

jsr     putsLCD

jsr     print_out

jsr     de2s

movb    #$0000,hold_count

brset   pbtn_status,#$02,*

rts

clear_outflg:

bclr    intFLG,#$08

rts

;PRINT_CMNDS.inc

;Lee Sichello

;purpose: subroutines for printing the instantaneous voltage/current/power and average power and daily cost

print_v:        ldx     #str_voltage

jsr     putsLCD

jsr     print_out

print_v_back:

brclr   pbtn_status,#$0F,print_v_back2

rts

print_v_back2:  brclr   intFLG,#$C0,print_v_back     ;stay until print flag or no load goes high

ldaa    #$C0

jsr     cmd2LCD

brset   intFLG,#$08,load_v2

bclr    intFLG,#$40

brset   intFLG,#$10,NoLoad_v

ldd     v_inst1

bra     load_v_buff

load_v2:

bclr    intFLG,#$80

brset   intFLG,#$20,NoLoad_v

ldd     v_inst2

load_v_buff:

ldy     #v_buffer

jsr     prep_2dp_buff

ldx     #v_buffer

jsr     putsLCD

ldx     #str_Volts

jsr     putsLCD

jmp     print_v_back

NoLoad_v:       jsr     print_NoLoad

jmp     print_v_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_i:        ldx     #str_current

jsr     putsLCD

jsr     print_out

print_i_back:

brclr   pbtn_status,#$0F,print_i_back2

rts

print_i_back2:  brclr   intFLG,#$C0,print_i_back     ;stay until print flag or no load goes high

ldaa    #$C0

jsr     cmd2LCD

brset   intFLG,#$08,load_i2

bclr    intFLG,#$40

brset   intFLG,#$10,NoLoad_i

ldd     i_inst1

bra     load_i_buff

load_i2:

bclr    intFLG,#$80

brset   intFLG,#$20,NoLoad_i

ldd     i_inst2

load_i_buff:

ldy     #i_buffer

jsr     prep_2dp_buff

ldx     #i_buffer

jsr     putsLCD

ldx     #str_Amps

jsr     putsLCD

bclr    intFLG,#$40

jmp     print_i_back

NoLoad_i:       jsr     print_NoLoad

jmp     print_i_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_p:        ldx     #str_power

jsr     putsLCD

jsr     print_out

print_p_back:

brclr   pbtn_status,#$0F,print_p_back2

rts

print_p_back2:  brclr   intFLG,#$C0,print_p_back     ;stay until print flag or no load goes high

ldaa    #$C0

jsr     cmd2LCD

cacl_p12:       ldd     i_inst1                     ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     p_inst1

ldd     i_inst2                     ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     p_inst2

brset   intFLG,#$08,load_p2

bclr    intFLG,#$40

brset   intFLG,#$10,NoLoad_p

ldd     p_inst1

bra     load_p_buff

load_p2:

bclr    intFLG,#$80

brset   intFLG,#$20,NoLoad_p

ldd     p_inst2

load_p_buff:

ldy     #p_buffer

jsr     prep_3dp_buff

ldx     #p_buffer

jsr     putsLCD

ldx     #str_kW

jsr     putsLCD

bclr    intFLG,#$40

jmp     print_p_back

NoLoad_p:       jsr     print_NoLoad

jmp     print_i_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

printPave:      ldx     #str_Pave

jsr     putsLCD

jsr     print_out

printPav_back:

brclr   pbtn_status,#$0F,printPav_back2

rts

printPav_back2: brclr   intFLG,#$C0,printPav_back     ;stay until print flag or no load goes high

ldaa    #$C0

jsr     cmd2LCD

calc_Pav12:     ldd     Iave1                       ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave1

ldd     Iave2                      ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave2

brset   intFLG,#$08,loadPav2

;bclr    intFLG,#$40

;brset   intFLG,#$10,NoLoad_Pav

ldd     Pave1

bra     loadPave_buff

loadPav2:       ldd     Pave2

;bclr    intFLG,#$80

;brset   intFLG,#$20,NoLoad_Pav

loadPave_buff:

ldy     #Pave_buffer

jsr     prep_3dp_buff

ldx     #Pave_buffer

jsr     putsLCD

ldx     #str_kW

jsr     putsLCD

bclr    intFLG,#$40

jmp     printPav_back

NoLoad_Pav:    	jsr     print_NoLoad

jmp     printPav_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_daily:

ldaa	#$80

jsr	cmd2LCD

ldx     #str_daily

jsr     putsLCD

jsr     print_out

daily_back:

brclr   pbtn_status,#$0F,is_cost_set

rts

is_cost_set:

ldx	cost

cpx	#$0000

bne	daily_back2

ldaa	#$C0

jsr	cmd2LCD

ldx	#str_NoCost

jsr	putsLCD

bra	daily_back

daily_back2:    brclr   intFLG,#$C0,daily_back     ;stay until print flag or no load goes high

calc_daily:     ldd     Iave1                       ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave1

ldy     Pave1

ldd     #24

emul                                ;y:d contains ave power(w) * 24 hrs

ldy     cost

emul

ldx     #1000

ediv                                        ;y:d contains cost in Wh*(cents/kWh) = cents/K

sty     daily1

ldd     Iave2                      ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave2

ldd     #24

emul                                ;y:d contains ave power(w) * 24 hrs

ldy     cost

emul

ldx     #1000

ediv                                        ;y:d contains cost in Wh*(cents/kWh) = cents/K

sty     daily2

ldaa    #$C1

jsr     cmd2LCD

brset   intFLG,#$08,load_daily2

ldd     daily1

bra     ld_daily_buff

load_daily2:    ldd     daily2

ld_daily_buff:

cpd	#10

bge	outpt_daily

ldaa	#$C0

jsr	cmd2LCD

ldx	#str_below0

jsr	putsLCD

jmp	print_daily

outpt_daily:

ldy     #daily_buffer

jsr     prep_2dp_buff

ldx     #daily_buffer

jsr     putsLCD

ldx	#str_11spaces

jsr	putsLCD

ldaa    #$C0

jsr     cmd2LCD

ldaa	#$24

jsr	putcLCD

bclr    intFLG,#$40

jmp     daily_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_totE:	ldx     #str_tot

jsr     putsLCD

jsr     print_out

printtot_back:

brclr   pbtn_status,#$0F,printtot_back2

rts

printtot_back2: brclr   intFLG,#$C0,printtot_back     ;stay until print flag or no load goes high

ldaa    #$C0

jsr     cmd2LCD

calc_tot12:     ldd     Iave1                       ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave1

ldd	seconds1

emul				;d*y->y:d = energy in W*s

ldx	#3600

ediv				:y=total w*hrs

sty	totE1

ldd     Iave2                      ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave2

ldd	seconds2

emul				;d*y->y:d = energy in W*s

ldx	#3600

ediv				:y=total w*hrs

sty	totE2

brset   intFLG,#$08,loadtot2

;bclr    intFLG,#$40

;brset   intFLG,#$10,NoLoad_tot

ldd     totE1

bra     loadtot_buff

loadtot2:       ldd     totE2

;bclr    intFLG,#$80

;brset   intFLG,#$20,NoLoad_tot

loadtot_buff:

ldy     #totE_buffer

jsr     prep_3dp_buff

ldx     #totE_buffer

jsr     putsLCD

ldx     #str_kWh

jsr     putsLCD

bclr    intFLG,#$40

jmp     printtot_back

NoLoad_tot:    	jsr     print_NoLoad

jmp     printtot_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_totC:

ldaa	#$80

jsr	cmd2LCD

ldx     #str_totcost

jsr     putsLCD

jsr     print_out

printC_back:

brclr   pbtn_status,#$0F,is_cost_setC

rts

is_cost_setC:

ldx	cost

cpx	#$0000

bne	printC_back2

ldaa	#$C0

jsr	cmd2LCD

ldx	#str_NoCost

jsr	putsLCD

bra	printC_back

printC_back2:   brclr   intFLG,#$C0,printC_back     ;stay until print flag or no load goes high

calc_C12:       ldd     Iave1                       ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave1

ldd	seconds1

emul				;d*y->y:d = energy in W*s

ldx	#3600

ediv				:y=total w*hrs

sty	totE1

ldd	cost

emul				;y*d->y:d contains total cost in thousands of dollars

ldx	#1000

ediv				;y contains cost in cents

sty	totcost1

ldd     Iave2                      ;current in mA  in d

ldy     #110                        ;assumed voltage

emul                                ;d*y->y:d contains  power in mW

ldx     #1000

ediv                                ;d:y/x->y (y contains power in W)

sty     Pave2

ldd	seconds2

emul				;d*y->y:d = energy in W*s

ldx	#3600

ediv				:y=total w*hrs

sty	totE2

ldd	cost

emul				;y*d->y:d contains total cost in thousands of dollars

ldx	#1000

ediv				;y contains cost in cents

sty	totcost2

ldaa    #$C1

jsr     cmd2LCD

brset   intFLG,#$08,loadC2

;bclr    intFLG,#$40

;brset   intFLG,#$10,NoLoad_C

ldd     totcost1

bra     loadC_buff

loadC2:       	ldd     totcost2

;bclr    intFLG,#$80

;brset   intFLG,#$20,NoLoad_C

loadC_buff:

cpd	#10

bge	outpt_totC

ldaa	#$C0

jsr	cmd2LCD

ldx	#str_below0

jsr	putsLCD

jmp	print_totC

outpt_totC:

ldy     #totC_buffer

jsr     prep_2dp_buff

ldx     #totC_buffer

jsr     putsLCD

ldx	#str_11spaces

jsr	putsLCD

ldaa	#$C0

jsr     cmd2LCD

ldaa	#$24

jsr	putcLCD

bclr    intFLG,#$40

jmp     printC_back

NoLoad_C:    	jsr     print_NoLoad

jmp     printC_back

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_out:

ldaa    #$8E

jsr     cmd2LCD

ldaa    #$23

jsr     putcLCD

brset    intFLG,#$08,print_out2

print_out1:

ldaa    #$31

jsr     putcLCD

rts

print_out2:

ldaa    #$32

jsr     putcLCD

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

print_NoLoad:   ldx     #str_NoLoad

jsr     putsLCD

rts

;buff_prep.inc

;Lee Sichello

;prepares buffers with various amounts of decimal places in ASCII for LCD output

prep_1dp_buff:  pshx                    ;expects value to convert in d , beginning of 6 bit buffer address in y

pshy

pshd

ldx     #100           ;

idiv                    ;d/x->x , rem->d

pshd

cpx     #$0000

beq     div_10

xgdx

addb    #$30          ;convert d to ascii

stab    0,Y

iny

div_10:

puld

ldx     #10            ;

idiv                    ;d/x->x , rem->d

pshd

xgdx

addb    #$30          ;convert d to ascii

stab    0,Y

ldaa    #46                    ;decimal point

staa    1,Y

puld

addb    #$30

stab    2,Y

ldaa    #$20                    ;space character

staa    3,Y

ldaa    #$00                    ;null character

staa    4,Y

puld

puly

pulx

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

prep_2dp_buff:  pshx                    ;expects value to convert in d , beginning of 6 bit buffer address in y

pshy

pshd

ldx     #10000           ;

idiv                    ;d/x->x , rem->d

pshd

cpx     #$0000

beq     div_1000

xgdx

addb    #$30          ;convert d to ascii

stab    0,Y

iny

div_1000:

puld

ldx     #1000            ;

idiv                    ;d/x->x , rem->d

pshd

xgdx

addb    #$30          ;convert d to ascii

stab    0,Y

ldaa    #46                    ;decimal point

staa    1,Y

puld

ldx     #100

idiv                  ;(10xrem)/51->x (first didgit of fraction), rem->d

pshd

xgdx

addb    #$30

stab    2,Y

puld

ldx     #10

idiv

xgdx

addb    #$30

stab    3,Y

ldaa    #$20                    ;space character

staa    4,Y

ldaa    #$00                    ;null character

staa    5,Y

puld

puly

pulx

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

prep_3dp_buff:  pshx                    ;expects value to convert in d , beginning of 6 bit buffer address in y

pshy

pshd

ldx     #1000            ;

idiv                    ;d/x->x , rem->d

pshd

xgdx

addb    #$30          ;convert d to ascii

stab    0,Y

ldaa    #46                    ;decimal point

staa    1,Y

puld

ldx     #100

idiv                  ;(10xrem)/51->x (first didgit of fraction), rem->d

pshd

xgdx

addb    #$30

stab    2,Y

puld

ldx     #10

idiv

pshd

xgdx

addb    #$30

stab    3,Y

puld

addb    #$30

stab    4,Y

ldaa    #$20                    ;space character

staa    5,Y

ldaa    #$00                    ;null character

staa    6,Y

puld

puly

pulx

rts

;LCD_cmnds.inc

lcd_dat:        equ     PORTK

lcd_dir:        equ     DDRK

lcd_E           equ     $02

lcd_RS          equ     $01

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

clear_LCD:      psha

ldaa    #$01

jsr     cmd2LCD

pula

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

cmd2LCD:        pshy

psha

psha

bclr    lcd_dat,lcd_RS

bset    lcd_dat,lcd_E

anda    #$F0

lsra

lsra

oraa    #lcd_E

staa    lcd_dat

nop

nop

nop

bclr    lcd_dat,lcd_E

pula

anda    #$0F

lsla

lsla

bset    lcd_dat,lcd_E

oraa    #lcd_E

staa    lcd_dat

nop

nop

nop

bclr    lcd_dat,lcd_E

ldy     #1

jsr     de50us

pula

puly

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

openLCD:        pshy

psha

movb    #$FF,lcd_dir

ldy     #10

jsr     de10ms

ldaa    #$28

jsr     cmd2lcd

ldaa    #$0F

jsr     cmd2lcd

ldaa    #$06

jsr     cmd2lcd

ldaa    #$01

jsr     cmd2lcd

ldy     #2

jsr     de10ms

pula

puly

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

putcLCD:        pshy

psha

psha

bset    lcd_dat,lcd_RS

bset    lcd_dat,lcd_E

anda    #$F0

lsra

lsra

oraa    #$03

staa    lcd_dat

nop

nop

nop

bclr    lcd_dat,lcd_E

pula

anda    #$0F

lsla

lsla

bset    lcd_dat,lcd_E

oraa    #$03

staa    lcd_dat

nop

nop

nop

bclr    lcd_dat,lcd_E

ldy     #1

jsr     de50us

pula

puly

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

putsLCD:

ldaa    1,x+

beq     donePS

jsr     putcLCD

bra     putsLCD

donePS:         rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

de10ms:         ldaa    #10                 ; delay 10 ms

staa    counter

de10ms1:        ldy     #6000                ; 6000 x 4 = 24,000 cycles = 1ms

de10ms2:        dey                          ; this instruction  takes 1 cycle

bne     de10ms2               ; this instruction  takes 3 cycles

dec     counter

bne     de10ms1               ; not 250ms yet, delay again

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

de50us:         ldaa    #250                  ; delay 250 us

staa    counter

de50us1:        ldy     #6                   ; 6 x 4 = 24 cycles = 1us

de50us2:        dey                          ; this instruction  takes 1 cycle

bne     de50us2                ; this instruction  takes 3 cycles

dec     counter

bne     de50us1               ; not 250ms yet, delay again

rts

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

de2s:         	ldaa    #50                  ; delay 250 us

staa    counter

de2s1:        	ldy     #60000                ; 60000 x 4 = 240000 cycles = 0.1s

de2s2:        	dey                          ; this instruction  takes 1 cycle

bne     de2s2                ; this instruction  takes 3 cycles

dec     counter

bne     de2s1               ; not 250ms yet, delay again

rts

;init_hrwr.inc

;hardware initialization for adc and pushbuttons

init_hrdwr:

movb    #$00,INITRG          ;configure registers(lab 6 handout)

movb    #$11,INITRM

bclr    CLKSEL,#$80          ;configure clock (lab 6 handout)

bset    PLLCTL,#$40

movb    #$2,SYNR

movb    #$0,REFDV

nop

nop

bclr    CRGFLG,#$08,*

bset    CLKSEL,#$80

ldx     #REGBLK

ldaa    #$ff

staa    ddrj,x               ; make port J an output port

staa    ddrb,x               ; make port B an output port

staa    ddrp,x               ; make port P an output port

staa    ptp,x                ; turn off 7-segment LED display

movb    #$FF,ptp                ; turn off 7-segment LED display

jsr     openLCD                 ; turn on/enable LCD screen

movb    #$00,DDRH

ldaa	#$0C

jsr	cmd2LCD

movb    #$C0,atd0ctl2           ;configure adc registers

movb    #$08,atd0ctl3

movb    #$05,atd0ctl4

;init_intrpt.inc

;rti interrupts initialized here

init_intrpt:

movb    #%01000000,RTICTL       ;set RTICLOCK to 2^13osclks , or 1ms

bset    CRGINT,#$80            ;enable RTI locally

cli                         ;enable interrupt globally

;RTI_ISR.inc

;Lee Sichello

;interrupt service routine that samples pushbuttons and current

RTI_ISR:        bset    CRGFLG,#$80

;---------------------------------------

TIMING_STUFF:

ldx	tcount1

inx

stx	tcount1

cpx	#1000

bne	TIMING2

movw	#$0000,tcount1

ldx	seconds1

inx

stx	seconds1

TIMING2:

ldx	tcount2

inx

stx	tcount2

cpx	#1000

bne	PGM_STUFF

movw	#$0000,tcount2

ldx	seconds2

inx

stx	seconds2

;---------------------------------------

PGM_STUFF:

ldx     pgm_count

cpx     #5000

bne     inc_pgm_count

movw    #$0000,pgm_count

bset    intFLG,#$03           ;set the timeout bit

jmp     CHECK_RST_HOLD

inc_pgm_count:

inx

stx     pgm_count

jmp     CHECK_RST_HOLD

;---------------------------------------

CHECK_RST_HOLD:

ldy     hold_count

iny

sty     hold_count

ldaa    pth                   ;retrieve pushbttn status in bits 0-3 (0=pressed)

coma                          ;1 = pressed

anda    #$0F                  ;clear upper bits

staa    pbtn_sample

brclr   pbtn_sample,#$02,PBTN_STUFF

movb    #$02,pbtn_status

done_RST:       rti

;---------------------------------------

PBTN_STUFF:

cmpa    #$00

beq     nothing_pressed      ;if no button is down, go to nonthing pressed

ldx     #750

cpx     pbtn_count

bne     low_pbtn_count     ;if pbtn_cout != 500, go to nothing pressed

bra     pbtn_pressed

nothing_pressed:

movb    #$00,pbtn_status

ldx     #750

stx     pbtn_count

bra     ADC_STUFF

low_pbtn_count:

ldx     pbtn_count

inx

stx     pbtn_count

bra     ADC_STUFF

pbtn_pressed:

staa    pbtn_status

movw    #$00,pbtn_count

jmp     ADC_STUFF

;--------------------------------------

ADC_STUFF:

;========

adc_out1:

movb    #$86,atd0ctl5           ;tells adc to take reading on an6

brclr   atd0stat0,#$80,*         ;wait until reading is finished

ldaa    atd0dr0h                 ;retrieve sample from data register

ldab    atd0dr0l                ;get upper half of sample into D

cpd     #5

bge     keep_sample1             ;if sample is greater than or = #$01

null_sample1:   ldx     n_nullsamp1

inx

stx     n_nullsamp1

ldx     #1000

cpx     n_nullsamp1

ble     no_load1                ;if 40 of 50 samples are null, there is no load

jmp     adc_out2

;set v_inst = 0

no_load1:       movw    #$0000,v_inst1

movw    #$0000,i_inst1

movw    #$0000,n_nullsamp1

bset    intFLG,#$10

bset    intFLG,#$40           ;set print flag

jmp     calc_Iave1

keep_sample1:   addd    sum_samp1                 ;add to sum of samples

std     sum_samp1                ;store new sum

ldx     n_samp1                  ;increment sample count

inx

stx     n_samp1

ldx     #50

cpx     n_samp1

ble     get_ave1                  ;if n_samples > 50 return get_ave

jmp     adc_out2

get_ave1:       ldd     sum_samp1                ;if n_samples >= 50 take sum/count=average

idiv                            ;d/x->x where x = average for 50 samples

xgdx                            ;put average into d

addd    sum_ave1               ;add 50cycle ave to sum

std     sum_ave1

ldx     n_ave1                   ;increment sample count

inx

stx     n_ave1

movw    #$0000,n_samp1             ;reset the cycle count

movw    #$0000,sum_samp1

movw    #$0000,n_nullsamp1

bclr    intFLG,#$10

ldx     #10

cpx     n_ave1

ble     get_v_inst1              ;if n_samples > 50 return get_ave

jmp     adc_out2

get_v_inst1:    ldd     sum_ave1                ;if n_samples >= 40 take sum/count=average

idiv                            ;d/x->x where x = average for 2000 samples

xgdx                            ;put average into d

ldy     #1000

emul                            ;d*1000 into y:d

ldx     #205

ediv                            ;y:d/x into Y, remainder into D (y=voltage in mV)

sty     v_inst1

ldd     #357                   ;conversion factor times 100

emul                           ;v_inst*395 into y:d  (result fits in d though)

ldx     #100

ediv                            ;v_vinst*395/100 into y  gives current (in mA)

sty     i_inst1

movw    #$0000,n_ave1             ;reset the cycle count

movw    #$0000,sum_ave1

bset    intFLG,#$40             ;set the intFLG

calc_Iave1:

ldd     Iave1

ldy     n_Iave1

emul                            ;Iave*n_Iave->y:d  (weighting)

addd    i_inst1                 ;add new entry to product

bcc     no_carry1               ;incremement y if needed

iny

no_carry1:      ldx     n_Iave1

inx

stx     n_Iave1

ediv                            ;[(old ave)*entries + new instantaneous]/entries+1 -> y

sty     Iave1

;=========

adc_out2:

movb    #$00,atd0dr0h

movb    #$00,atd0dr0h

movb    #$82,atd0ctl5           ;tells adc to take reading on an5

brclr   atd0stat0,#$80,*         ;wait until reading is finished

ldaa    atd0dr0h                 ;retrieve sample from data register

ldab    atd0dr0l                ;get upper half of sample into D

cpd     #5

bge     keep_sample2             ;if sample is greater than or = #$01

null_sample2:   ldx     n_nullsamp2

inx

stx     n_nullsamp2

ldx     #1000

cpx     n_nullsamp2

ble     no_load2                ;if 40 of 50 samples are null, there is no load

jmp     done_adc

;set v_inst = 0

no_load2:       movw    #$0000,v_inst2

movw    #$0000,i_inst2

movw    #$0000,n_nullsamp2

bset    intFLG,#$20

bset    intFLG,#$80           ;set print flag

jmp     calc_Iave2

keep_sample2:   addd    sum_samp2                 ;add to sum of samples

std     sum_samp2                ;store new sum

ldx     n_samp2                  ;increment sample count

inx

stx     n_samp2

ldx     #50

cpx     n_samp2

ble     get_ave2                  ;if n_samples > 50 return get_ave

jmp     done_adc

get_ave2:       ldd     sum_samp2                ;if n_samples >= 50 take sum/count=average

idiv                            ;d/x->x where x = average for 50 samples

xgdx                            ;put average into d

addd    sum_ave2               ;add 50cycle ave to sum

std     sum_ave2

ldx     n_ave2                   ;increment sample count

inx

stx     n_ave2

movw    #$0000,n_samp2             ;reset the cycle count

movw    #$0000,sum_samp2

movw    #$0000,n_nullsamp2

bclr    intFLG,#$20

ldx     #10

cpx     n_ave2

ble     get_v_inst2              ;if n_samples > 50 return get_ave

jmp     done_adc

get_v_inst2:    ldd     sum_ave2                ;if n_samples >= 40 take sum/count=average

idiv                            ;d/x->x where x = average for 2000 samples

xgdx                            ;put average into d

ldy     #1000

emul                            ;d*1000 into y:d

ldx     #205

ediv                            ;y:d/x into Y, remainder into D (y=voltage in mV)

sty     v_inst2

ldd     #379                   ;conversion factor times 100

emul                           ;v_inst*395 into y:d  (result fits in d though)

ldx     #100

ediv                            ;v_vinst*395/100 into y  gives current (in mA)

sty     i_inst2

movw    #$0000,n_ave2             ;reset the cycle count

movw    #$0000,sum_ave2

bset    intFLG,#$80             ;set the intFLG

calc_Iave2:

ldd     Iave2

ldy     n_Iave2

emul                            ;Iave*n_Iave->y:d  (weighting)

addd    i_inst2                 ;add new entry to product

bcc     no_carry2               ;incremement y if needed

iny

no_carry2:      ldx     n_Iave2

inx

stx     n_Iave2

ediv                            ;[(old ave)*entries + new instantaneous]/entries+1 -> y

sty     Iave2

done_adc:       rti

;DATA.inc

;Lee Sichello

;A set of strings for LCD output

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

str_pgm:        fcc     'SET LOCAL COST '

fcb     $00

str_cents:      fcc     'cents/kWh   '

fcb     $00

str_voltage:    fcc     'VOLTAGE '

fcb     $00

str_current:    fcc     'CURRENT '

fcb     $00

str_power:      fcc     'POWER '

fcb     $00

str_Pave:       fcc     'AVERAGE POWER '

fcb     $00

str_daily:      fcc     'DAILY COST '

fcb     $00

str_$:          fcc     '$'

fcb     $00

str_Amps:       fcc     'Amps            '

fcb     $00

str_Volts:      fcc     'Volts           '

fcb     $00

str_kWh         fcc     'kWh             '

fcb     $00

str_kW:         fcc     'kW              '

fcb     #$00

str_NoLoad:     fcc     '**NO LOAD**'

fcb     $00

str_rst:        fcc     'RESET OUTLET '

fcb     $00

str_NoCost:	fcc	'Must Set $/kWh  '

fcb	$00

str_below0:	fcc	'Under 1 cent '

fcb	$00

str_tot:	fcc	'TOTAL ENERGY'

fcb	$00

str_11spaces:	fcc	'           '

fcb	$00

str_totcost:	fcc	'TOTAL COST'

fcb	$00

; D_variant_registers.inc

; Register addresses in this file can be relocated by setting REGBASE to a valid offset.

;(YOU MUST ACTUALLY RELOCATE THE 9S12 REGISTER BLOCK for this to work)

REGBASE        equ $0

*

*

*  HC12 i/o register locations (Not all subsystems exist in all D versions)

*

*

PORTA          equ REGBASE+0   ;port a = address lines a8 - a15

PORTB          equ REGBASE+1   ;port b = address lines a0 - a7

DDRA           equ REGBASE+2   ;port a direction register

DDRB           equ REGBASE+3   ;port b direction register

PORTE          equ REGBASE+8   ;port e = mode,irq and control signals

DDRE           equ REGBASE+9   ;port e direction register

PEAR           equ REGBASE+$a  ;port e assignments

MODE           equ REGBASE+$b  ;mode register

PUCR           equ REGBASE+$c  ;port pull-up control register

RDRIV          equ REGBASE+$d  ;port reduced drive control register

EBICTL         equ REGBASE+$e  ;e stretch control

INITRM         equ REGBASE+$10 ;ram location register

INITRG         equ REGBASE+$11 ;register location register

INITEE         equ REGBASE+$12 ;eeprom location register

MISC           equ REGBASE+$13 ;miscellaneous mapping control

MTST0          equ REGBASE+$14 ; reserved

ITCR           equ REGBASE+$15 ;interrupt test control register

ITEST          equ REGBASE+$16 ;interrupt test register

MTST1          equ REGBASE+$17 ; reserved

PARTIDH        equ REGBASE+$1a ;part id high

PARTIDL        equ REGBASE+$1b ;part id low

MEMSIZ0        equ REGBASE+$1c ;memory size

MEMSIZ1        equ REGBASE+$1d ;memory size

IRQCR          equ REGBASE+$1e ;interrupt control register

INTCR          equ REGBASE+$1e ;interrupt control register (old name)

HPRIO          equ REGBASE+$1f ;high priority reg

BKPCT0         equ REGBASE+$28 ;break control register

BKPCT1         equ REGBASE+$29 ;break control register

BKP0X          equ REGBASE+$2a ; break 0 index register

BKP0H          equ REGBASE+$2b ; break 0 pointer high

BRP0L          equ REGBASE+$2c ; break 0 pointer low

BKP1X          equ REGBASE+$2d ; break 1 index register

BKP1H          equ REGBASE+$2e ; break 1 pointer high

BRP1L          equ REGBASE+$2f ; break 1 pointer low

PPAGE          equ REGBASE+$30 ;program page register

PORTK          equ REGBASE+$32 ;port k data

DDRK           equ REGBASE+$33 ;port k direction

SYNR           equ REGBASE+$34 ; synthesizer / multiplier register

REFDV          equ REGBASE+$35 ; reference divider register

CTFLG          equ REGBASE+$36 ; reserved

CRGFLG         equ REGBASE+$37 ; pll flags register

CRGINT         equ REGBASE+$38 ; pll interrupt register

CLKSEL         equ REGBASE+$39 ; clock select register

PLLCTL         equ REGBASE+$3a ; pll control register

RTICTL         equ REGBASE+$3b ;real time interrupt control

COPCTL         equ REGBASE+$3c ;watchdog control

FORBYP         equ REGBASE+$3d ;

CTCTL          equ REGBASE+$3e ;

ARMCOP         equ REGBASE+$3f ;cop reset register

TIOS           equ REGBASE+$40 ;timer input/output select

CFORC          equ REGBASE+$41 ;timer compare force

OC7M           equ REGBASE+$42 ;timer output compare 7 mask

OC7D           equ REGBASE+$43 ;timer output compare 7 data

TCNT           equ REGBASE+$44 ;timer counter register hi

*TCNT          equ REGBASE+$45 ;timer counter register lo

TSCR           equ REGBASE+$46 ;timer system control register (Old Name)

TSCR1          equ REGBASE+$46 ;timer system control register

TTOV           equ REGBASE+$47 ;reserved

TCTL1          equ REGBASE+$48 ;timer control register 1

TCTL2          equ REGBASE+$49 ;timer control register 2

TCTL3          equ REGBASE+$4a ;timer control register 3

TCTL4          equ REGBASE+$4b ;timer control register 4

TMSK1          equ REGBASE+$4c ;timer interrupt mask 1 (Old Name)

TIE            equ REGBASE+$4c ;timer interrupt mask 1

TMSK2          equ REGBASE+$4d ;timer interrupt mask 2 (Old Name)

TSCR2          equ REGBASE+$4d ;timer interrupt mask 2

TFLG1          equ REGBASE+$4e ;timer flags 1

TFLG2          equ REGBASE+$4f ;timer flags 2

TC0            equ REGBASE+$50 ;timer capture/compare register 0

*TC0           equ REGBASE+$51 ; low byte

TC1            equ REGBASE+$52 ;timer capture/compare register 1

*TC1           equ REGBASE+$53 ;; low byte

TC2            equ REGBASE+$54 ;timer capture/compare register 2

*TC2           equ REGBASE+$55 ; low byte

TC3            equ REGBASE+$56 ;timer capture/compare register 3

*TC3           equ REGBASE+$57 ; low byte

TC4            equ REGBASE+$58 ;timer capture/compare register 4

*TC4           equ REGBASE+$59 ; low byte

TC5            equ REGBASE+$5a ;timer capture/compare register 5

*TC5           equ REGBASE+$5b ; low byte

TC6            equ REGBASE+$5c ;timer capture/compare register 6

*TC6           equ REGBASE+$5d ; low byte

TC7            equ REGBASE+$5e ;timer capture/compare register 7

*TC7           equ REGBASE+$5f ; low byte

PACTL          equ REGBASE+$60 ;pulse accumulator controls

PAFLG          equ REGBASE+$61 ;pulse accumulator flags

PACN3          equ REGBASE+$62 ;pulse accumulator counter 3

PACN2          equ REGBASE+$63 ;pulse accumulator counter 2

PACN1          equ REGBASE+$64 ;pulse accumulator counter 1

PACN0          equ REGBASE+$65 ;pulse accumulator counter 0

MCCTL          equ REGBASE+$66 ;modulus down conunter control

MCFLG          equ REGBASE+$67 ;down counter flags

ICPAR          equ REGBASE+$68 ;input pulse accumulator control

DLYCT          equ REGBASE+$69 ;delay count to down counter

ICOVW          equ REGBASE+$6a ;input control overwrite register

ICSYS          equ REGBASE+$6b ;input control system control

TIMTST         equ REGBASE+$6d ;timer test register

PBCTL          equ REGBASE+$70 ; pulse accumulator b control

PBFLG          equ REGBASE+$71 ; pulse accumulator b flags

PA3H           equ REGBASE+$72 ; pulse accumulator holding register 3

PA2H           equ REGBASE+$73 ; pulse accumulator holding register 2

PA1H           equ REGBASE+$74 ; pulse accumulator holding register 1

PA0H           equ REGBASE+$75 ; pulse accumulator holding register 0

MCCNT          equ REGBASE+$76 ; modulus down counter register

*MCCNTL        equ REGBASE+$77 ; low byte

TCOH           equ REGBASE+$78 ; capture 0 holding register

*TCOH          equ REGBASE+$79 ; low byte

TC1H           equ REGBASE+$7a ; capture 1 holding register

*TC1H          equ REGBASE+$7b ; low byte

TC2H           equ REGBASE+$7c ; capture 2 holding register

*TC2H          equ REGBASE+$7d ; low byte

TC3H           equ REGBASE+$7e ; capture 3 holding register

*TC3H          equ REGBASE+$7f ; low byte

ATD0CTL0       equ REGBASE+$80 ;adc control 0 (reserved)

ATD0CTL1       equ REGBASE+$81 ;adc control 1 (reserved)

ATD0CTL2       equ REGBASE+$82 ;adc control 2

ATD0CTL3       equ REGBASE+$83 ;adc control 3

ATD0CTL4       equ REGBASE+$84 ;adc control 4

ATD0CTL5       equ REGBASE+$85 ;adc control 5

ATD0STAT0      equ REGBASE+$86 ;adc status register 0

ATD0TEST0      equ REGBASE+$88 ;adc test (reserved)

ATD0TEST1      equ REGBASE+$89 ;adc test (reserved)

ATD0STAT1      equ REGBASE+$8b ;adc status register 1

ATD0DIEN       equ REGBASE+$8d ;

PORTAD0        equ REGBASE+$8f ;port adc = input only

ATD0DR0H       equ REGBASE+$90 ;adc result 0 register

ATD0DR0L       equ REGBASE+$91 ; low byte

ATD0DR1H       equ REGBASE+$92 ;adc result 1 register

ATD0DR1L       equ REGBASE+$93 ; low byte

ATD0DR2H       equ REGBASE+$94 ;adc result 2 register

ATD0DR2L       equ REGBASE+$95 ; low byte

ATD0DR3H       equ REGBASE+$96 ;adc result 3 register

ATD0DR3L       equ REGBASE+$97 ; low byte

ATD0DR4H       equ REGBASE+$98 ;adc result 4 register

ATD0DR4L       equ REGBASE+$99 ; low byte

ATD0DR5H       equ REGBASE+$9a ;adc result 5 register

ATD0DR5L       equ REGBASE+$9B ; low byte

ATD0DR6H       equ REGBASE+$9c ;adc result 6 register

ATD0DR6L       equ REGBASE+$9D ; low byte

ATD0DR7H       equ REGBASE+$9e ;adc result 7 register

ATD0DR7L       equ REGBASE+$9F ; low byte

PWME           equ REGBASE+$a0 ;pwm enable

PWMPOL         equ REGBASE+$a1 ;pwm polarity

PWMCLK         equ REGBASE+$a2 ;pwm clock select register

PWMPRCLK       equ REGBASE+$a3 ;pwm prescale clock select register

PWMCAE         equ REGBASE+$a4 ;pwm center align select register

PWMCTL         equ REGBASE+$a5 ;pwm control register

PWMTST         equ REGBASE+$a6 ;reserved

PWMPRSC        equ REGBASE+$a7 ;reserved

PWMSCLA        equ REGBASE+$a8 ;pwm scale a

PWMSCLB        equ REGBASE+$a9 ;pwm scale b

PWMSCNTA       equ REGBASE+$aa ;reserved

PWMSCNTB       equ REGBASE+$ab ;reserved

PWMCNT0        equ REGBASE+$ac ;pwm channel 0 counter

PWMCNT1        equ REGBASE+$ad ;pwm channel 1 counter

PWMCNT2        equ REGBASE+$ae ;pwm channel 2 counter

PWMCNT3        equ REGBASE+$af ;pwm channel 3 counter

PWMCNT4        equ REGBASE+$b0 ;pwm channel 4 counter

PWMCNT5        equ REGBASE+$b1 ;pwm channel 5 counter

PWMCNT6        equ REGBASE+$b2 ;pwm channel 6 counter

PWMCNT7        equ REGBASE+$b3 ;pwm channel 7 counter

PWMPER0        equ REGBASE+$b4 ;pwm channel 0 period

PWMPER1        equ REGBASE+$b5 ;pwm channel 1 period

PWMPER2        equ REGBASE+$b6 ;pwm channel 2 period

PWMPER3        equ REGBASE+$b7 ;pwm channel 3 period

PWMPER4        equ REGBASE+$b8 ;pwm channel 4 period

PWMPER5        equ REGBASE+$b9 ;pwm channel 5 period

PWMPER6        equ REGBASE+$ba ;pwm channel 6 period

PWMPER7        equ REGBASE+$bb ;pwm channel 7 period

PWMDTY0        equ REGBASE+$bc ;pwm channel 0 duty cycle

PWMDTY1        equ REGBASE+$bd ;pwm channel 1 duty cycle

PWMDTY2        equ REGBASE+$be ;pwm channel 2 duty cycle

PWMDTY3        equ REGBASE+$bf ;pwm channel 3 duty cycle

PWMDTY4        equ REGBASE+$c0 ;pwm channel 4 duty cycle

PWMDTY5        equ REGBASE+$c1 ;pwm channel 5 duty cycle

PWMDTY6        equ REGBASE+$c2 ;pwm channel 6 duty cycle

PWMDTY7        equ REGBASE+$c3 ;pwm channel 7 duty cycle

PWMSDN         equ REGBASE+$c4 ;pwm shutdown register

SCI0BDH        equ REGBASE+$c8 ;sci 0 baud reg hi byte

SCI0BDL        equ REGBASE+$c9 ;sci 0 baud reg lo byte

SCI0CR1        equ REGBASE+$ca ;sci 0 control1 reg

SCI0CR2        equ REGBASE+$cb ;sci 0 control2 reg

SCI0SR1        equ REGBASE+$cc ;sci 0 status reg 1

SCI0SR2        equ REGBASE+$cd ;sci 0 status reg 2

SCI0DRH        equ REGBASE+$ce ;sci 0 data reg hi

SCI0DRL        equ REGBASE+$cf ;sci 0 data reg lo

SCI1BDH        equ REGBASE+$d0 ;sci 1 baud reg hi byte

SCI1BDL        equ REGBASE+$d1 ;sci 1 baud reg lo byte

SCI1CR1        equ REGBASE+$d2 ;sci 1 control1 reg

SCI1CR2        equ REGBASE+$d3 ;sci 1 control2 reg

SCI1SR1        equ REGBASE+$d4 ;sci 1 status reg 1

SCI1SR2        equ REGBASE+$d5 ;sci 1 status reg 2

SCI1DRH        equ REGBASE+$d6 ;sci 1 data reg hi

SCI1DRL        equ REGBASE+$d7 ;sci 1 data reg lo

SPI0CR1        equ REGBASE+$d8 ;spi 0 control1 reg

SPI0CR2        equ REGBASE+$d9 ;spi 0 control2 reg

SPI0BR         equ REGBASE+$da ;spi 0 baud reg

SPI0SR         equ REGBASE+$db ;spi 0 status reg hi

SPI0DR         equ REGBASE+$dd ;spi 0 data reg

IBAD           equ REGBASE+$e0 ;i2c bus address register

IBFD           equ REGBASE+$e1 ;i2c bus frequency divider

IBCR           equ REGBASE+$e2 ;i2c bus control register

IBSR           equ REGBASE+$e3 ;i2c bus status register

IBDR           equ REGBASE+$e4 ;i2c bus message data register

DLCBCR1        equ REGBASE+$e8 ;bdlc control regsiter 1

DLCBSVR        equ REGBASE+$e9 ;bdlc state vector register

DLCBCR2        equ REGBASE+$ea ;bdlc control register 2

DLCBDR         equ REGBASE+$eb ;bdlc data register

DLCBARD        equ REGBASE+$ec ;bdlc analog delay register

DLCBRSR        equ REGBASE+$ed ;bdlc rate select register

DLCSCR         equ REGBASE+$ee ;bdlc control register

DLCBSTAT       equ REGBASE+$ef ;bdlc status register

SPI1CR1        equ REGBASE+$f0 ;spi 1 control1 reg

SPI1CR2        equ REGBASE+$f1 ;spi 1 control2 reg

SPI1BR         equ REGBASE+$f2 ;spi 1 baud reg

SPI1SR         equ REGBASE+$f3 ;spi 1 status reg hi

SPI1DR         equ REGBASE+$f5 ;spi 1 data reg

SPI2CR1        equ REGBASE+$f8 ;spi 2 control1 reg

SPI2CR2        equ REGBASE+$f9 ;spi 2 control2 reg

SPI2BR         equ REGBASE+$fa ;spi 2 baud reg

SPI2SR         equ REGBASE+$fb ;spi 2 status reg hi

SPI2DR         equ REGBASE+$fd ;spi 2 data reg

FCLKDIV        equ REGBASE+$100 ;flash clock divider

FSEC           equ REGBASE+$101 ;flash security register

FTSTMOD        equ REGBASE+$102 ;TEST register

FCNFG          equ REGBASE+$103 ;flash configuration register

FPROT          equ REGBASE+$104 ;flash protection register

FSTAT          equ REGBASE+$105 ;flash status register

FCMD           equ REGBASE+$106 ;flash command register

FADDRHI        equ REGBASE+$108 ;flash addr register hi

FADDRLO        equ REGBASE+$109 ;flash addr register lo

FDATAHI        equ REGBASE+$10a ;flash data register hi

FDATALO        equ REGBASE+$10b ;flash data register lo

ECLKDIV        equ REGBASE+$110 ;eeprom clock divider

ECNFG          equ REGBASE+$113 ;eeprom configuration register

EPROT          equ REGBASE+$114 ;eeprom protection register

ESTAT          equ REGBASE+$115 ;eeprom status register

ECMD           equ REGBASE+$116 ;eeprom command register

EADDRHI        equ REGBASE+$118 ;eeprom addr register hi

EADDRLO        equ REGBASE+$119 ;eeprom addr register lo

EDATAHI        equ REGBASE+$11a ;eeprom data register hi

EDATALO        equ REGBASE+$11b ;eeprom data register lo

ATD1CTL0       equ REGBASE+$120 ;adc1 control 0 (reserved)

ATD1CTL1       equ REGBASE+$121 ;adc1 control 1 (reserved)

ATD1CTL2       equ REGBASE+$122 ;adc1 control 2

ATD1CTL3       equ REGBASE+$123 ;adc1 control 3

ATD1CTL4       equ REGBASE+$124 ;adc1 control 4

ATD1CTL5       equ REGBASE+$125 ;adc1 control 5

ATD1STAT0      equ REGBASE+$126 ;adc1 status register hi

ATD1TEST0      equ REGBASE+$128 ;adc1 test (reserved)

ATD1TEST1      equ REGBASE+$129 ;adc1 test (reserved)

ATD1STAT1      equ REGBASE+$12b ;adc1 status register lo

ATD1DIEN       equ REGBASE+$12d ;adc1 input enable register

PORTAD1        equ REGBASE+$12f ;port adc1 = input only

ATD1DR0H       equ REGBASE+$130 ;adc1 result 0 register

ATD1DR0L       equ REGBASE+$131 ;low byte

ATD1DR1H       equ REGBASE+$132 ;adc1 result 1 register

ATD1DR1L       equ REGBASE+$133 ;low byte

ATD1DR2H       equ REGBASE+$134 ;adc1 result 2 register

ATD1DR2L       equ REGBASE+$135 ;low byte

ATD1DR3H       equ REGBASE+$136 ;adc1 result 3 register

ATD1DR3L       equ REGBASE+$137 ;low byte

ATD1DR4H       equ REGBASE+$138 ;adc1 result 4 register

ATD1DR4L       equ REGBASE+$139 ;low byte

ATD1DR5H       equ REGBASE+$13a ;adc1 result 5 register

ATD1DR5L       equ REGBASE+$13b ;low byte

ATD1DR6H       equ REGBASE+$13c ;adc1 result 6 register

ATD1DR6L       equ REGBASE+$13d ;low byte

ATD1DR7H       equ REGBASE+$13e ;adc1 result 7 register

ATD1DR7L       equ REGBASE+$13f ;low byte

CAN0CTL0       equ REGBASE+$140 ;can0 control register 0

CAN0CTL1       equ REGBASE+$141 ;can0 control register 1

CAN0BTR0       equ REGBASE+$142 ;can0 bus timing register 0

CAN0BTR1       equ REGBASE+$143 ;can0 bus timing register 1

CAN0RFLG       equ REGBASE+$144 ;can0 receiver flags

CAN0RIER       equ REGBASE+$145 ;can0 receiver interrupt enables

CAN0TFLG       equ REGBASE+$146 ;can0 transmit flags

CAN0TIER       equ REGBASE+$147 ;can0 transmit interrupt enables

CAN0TARQ       equ REGBASE+$148 ;can0 transmit message abort control

CAN0TAAK       equ REGBASE+$149 ;can0 transmit message abort status

CAN0TBSEL      equ REGBASE+$14a ;can0 transmit buffer select

CAN0IDAC       equ REGBASE+$14b ;can0 identfier acceptance control

CAN0RXERR      equ REGBASE+$14e ;can0 receive error counter

CAN0TXERR      equ REGBASE+$14f ;can0 transmit error counter

CAN0IDAR0      equ REGBASE+$150 ;can0 identifier acceptance register 0

CAN0IDAR1      equ REGBASE+$151 ;can0 identifier acceptance register 1

CAN0IDAR2      equ REGBASE+$152 ;can0 identifier acceptance register 2

CAN0IDAR3      equ REGBASE+$153 ;can0 identifier acceptance register 3

CAN0IDMR0      equ REGBASE+$154 ;can0 identifier mask register 0

CAN0IDMR1      equ REGBASE+$155 ;can0 identifier mask register 1

CAN0IDMR2      equ REGBASE+$156 ;can0 identifier mask register 2

CAN0IDMR3      equ REGBASE+$157 ;can0 identifier mask register 3

CAN0IDAR4      equ REGBASE+$158 ;can0 identifier acceptance register 4

CAN0IDAR5      equ REGBASE+$159 ;can0 identifier acceptance register 5

CAN0IDAR6      equ REGBASE+$15a ;can0 identifier acceptance register 6

CAN0IDAR7      equ REGBASE+$15b ;can0 identifier acceptance register 7

CAN0IDMR4      equ REGBASE+$15c ;can0 identifier mask register 4

CAN0IDMR5      equ REGBASE+$15d ;can0 identifier mask register 5

CAN0IDMR6      equ REGBASE+$15e ;can0 identifier mask register 6

CAN0IDMR7      equ REGBASE+$15f ;can0 identifier mask register 7

CAN0RXFG       equ REGBASE+$160 ;can0 rx foreground buffer thru +$16f

CAN0TXFG       equ REGBASE+$170 ;can0 tx foreground buffer thru +$17f

CAN1CTL0       equ REGBASE+$180 ;can1 control register 0

CAN1CTL1       equ REGBASE+$181 ;can1 control register 1

CAN1BTR0       equ REGBASE+$182 ;can1 bus timing register 0

CAN1BTR1       equ REGBASE+$183 ;can1 bus timing register 1

CAN1RFLG       equ REGBASE+$184 ;can1 receiver flags

CAN1RIER       equ REGBASE+$185 ;can1 receiver interrupt enables

CAN1TFLG       equ REGBASE+$186 ;can1 transmit flags

CAN1TIER       equ REGBASE+$187 ;can1 transmit interrupt enables

CAN1TARQ       equ REGBASE+$188 ;can1 transmit message abort control

CAN1TAAK       equ REGBASE+$189 ;can1 transmit message abort status

CAN1TBSEL      equ REGBASE+$18a ;can1 transmit buffer select

CAN1IDAC       equ REGBASE+$18b ;can1 identfier acceptance control

CAN1RXERR      equ REGBASE+$18e ;can1 receive error counter

CAN1TXERR      equ REGBASE+$18f ;can1 transmit error counter

CAN1IDAR0      equ REGBASE+$190 ;can1 identifier acceptance register 0

CAN1IDAR1      equ REGBASE+$191 ;can1 identifier acceptance register 1

CAN1IDAR2      equ REGBASE+$192 ;can1 identifier acceptance register 2

CAN1IDAR3      equ REGBASE+$193 ;can1 identifier acceptance register 3

CAN1IDMR0      equ REGBASE+$194 ;can1 identifier mask register 0

CAN1IDMR1      equ REGBASE+$195 ;can1 identifier mask register 1

CAN1IDMR2      equ REGBASE+$196 ;can1 identifier mask register 2

CAN1IDMR3      equ REGBASE+$197 ;can1 identifier mask register 3

CAN1IDAR4      equ REGBASE+$198 ;can1 identifier acceptance register 4

CAN1IDAR5      equ REGBASE+$199 ;can1 identifier acceptance register 5

CAN1IDAR6      equ REGBASE+$19a ;can1 identifier acceptance register 6

CAN1IDAR7      equ REGBASE+$19b ;can1 identifier acceptance register 7

CAN1IDMR4      equ REGBASE+$19c ;can1 identifier mask register 4

CAN1IDMR5      equ REGBASE+$19d ;can1 identifier mask register 5

CAN1IDMR6      equ REGBASE+$19e ;can1 identifier mask register 6

CAN1IDMR7      equ REGBASE+$19f ;can1 identifier mask register 7

CAN1RXFG       equ REGBASE+$1a0 ;can1 rx foreground buffer thru +$1af

CAN1TXFG       equ REGBASE+$1b0 ;can1 tx foreground buffer thru +$1bf

CAN2CTL0       equ REGBASE+$1c0 ;can2 control register 0

CAN2CTL1       equ REGBASE+$1c1 ;can2 control register 1

CAN2BTR0       equ REGBASE+$1c2 ;can2 bus timing register 0

CAN2BTR1       equ REGBASE+$1c3 ;can2 bus timing register 1

CAN2RFLG       equ REGBASE+$1c4 ;can2 receiver flags

CAN2RIER       equ REGBASE+$1c5 ;can2 receiver interrupt enables

CAN2TFLG       equ REGBASE+$1c6 ;can2 transmit flags

CAN2TIER       equ REGBASE+$1c7 ;can2 transmit interrupt enables

CAN2TARQ       equ REGBASE+$1c8 ;can2 transmit message abort control

CAN2TAAK       equ REGBASE+$1c9 ;can2 transmit message abort status

CAN2TBSEL      equ REGBASE+$1ca ;can2 transmit buffer select

CAN2IDAC       equ REGBASE+$1cb ;can2 identfier acceptance control

CAN2RXERR      equ REGBASE+$1ce ;can2 receive error counter

CAN2TXERR      equ REGBASE+$1cf ;can2 transmit error counter

CAN2IDAR0      equ REGBASE+$1d0 ;can2 identifier acceptance register 0

CAN2IDAR1      equ REGBASE+$1d1 ;can2 identifier acceptance register 1

CAN2IDAR2      equ REGBASE+$1d2 ;can2 identifier acceptance register 2

CAN2IDAR3      equ REGBASE+$1d3 ;can2 identifier acceptance register 3

CAN2IDMR0      equ REGBASE+$1d4 ;can2 identifier mask register 0

CAN2IDMR1      equ REGBASE+$1d5 ;can2 identifier mask register 1

CAN2IDMR2      equ REGBASE+$1d6 ;can2 identifier mask register 2

CAN2IDMR3      equ REGBASE+$1d7 ;can2 identifier mask register 3

CAN2IDAR4      equ REGBASE+$1d8 ;can2 identifier acceptance register 4

CAN2IDAR5      equ REGBASE+$1d9 ;can2 identifier acceptance register 5

CAN2IDAR6      equ REGBASE+$1da ;can2 identifier acceptance register 6

CAN2IDAR7      equ REGBASE+$1db ;can2 identifier acceptance register 7

CAN2IDMR4      equ REGBASE+$1dc ;can2 identifier mask register 4

CAN2IDMR5      equ REGBASE+$1dd ;can2 identifier mask register 5

CAN2IDMR6      equ REGBASE+$1de ;can2 identifier mask register 6

CAN2IDMR7      equ REGBASE+$1df ;can2 identifier mask register 7

CAN2RXFG       equ REGBASE+$1e0 ;can2 rx foreground buffer thru +$1ef

CAN2TXFG       equ REGBASE+$1f0 ;can2 tx foreground buffer thru +$1ff

CAN3CTL0       equ REGBASE+$200 ;can3 control register 0

CAN3CTL1       equ REGBASE+$201 ;can3 control register 1

CAN3BTR0       equ REGBASE+$202 ;can3 bus timing register 0

CAN3BTR1       equ REGBASE+$203 ;can3 bus timing register 1

CAN3RFLG       equ REGBASE+$204 ;can3 receiver flags

CAN3RIER       equ REGBASE+$205 ;can3 receiver interrupt enables

CAN3TFLG       equ REGBASE+$206 ;can3 transmit flags

CAN3TIER       equ REGBASE+$207 ;can3 transmit interrupt enables

CAN3TARQ       equ REGBASE+$208 ;can3 transmit message abort control

CAN3TAAK       equ REGBASE+$209 ;can3 transmit message abort status

CAN3TBSEL      equ REGBASE+$20a ;can3 transmit buffer select

CAN3IDAC       equ REGBASE+$20b ;can3 identfier acceptance control

CAN3RXERR      equ REGBASE+$20e ;can3 receive error counter

CAN3TXERR      equ REGBASE+$20f ;can3 transmit error counter

CAN3IDAR0      equ REGBASE+$210 ;can3 identifier acceptance register 0

CAN3IDAR1      equ REGBASE+$211 ;can3 identifier acceptance register 1

CAN3IDAR2      equ REGBASE+$212 ;can3 identifier acceptance register 2

CAN3IDAR3      equ REGBASE+$213 ;can3 identifier acceptance register 3

CAN3IDMR0      equ REGBASE+$214 ;can3 identifier mask register 0

CAN3IDMR1      equ REGBASE+$215 ;can3 identifier mask register 1

CAN3IDMR2      equ REGBASE+$216 ;can3 identifier mask register 2

CAN3IDMR3      equ REGBASE+$217 ;can3 identifier mask register 3

CAN3IDAR4      equ REGBASE+$218 ;can3 identifier acceptance register 4

CAN3IDAR5      equ REGBASE+$219 ;can3 identifier acceptance register 5

CAN3IDAR6      equ REGBASE+$21a ;can3 identifier acceptance register 6

CAN3IDAR7      equ REGBASE+$21b ;can3 identifier acceptance register 7

CAN3IDMR4      equ REGBASE+$21c ;can3 identifier mask register 4

CAN3IDMR5      equ REGBASE+$21d ;can3 identifier mask register 5

CAN3IDMR6      equ REGBASE+$21e ;can3 identifier mask register 6

CAN3IDMR7      equ REGBASE+$21f ;can3 identifier mask register 7

CAN3RXFG       equ REGBASE+$220 ;can3 rx foreground buffer thru +$22f

CAN3TXFG       equ REGBASE+$230 ;can3 tx foreground buffer thru +$23f

PTT            equ REGBASE+$240 ;portt data register

PTIT           equ REGBASE+$241 ;portt input register

DDRT           equ REGBASE+$242 ;portt direction register

RDRT           equ REGBASE+$243 ;portt reduced drive register

PERT           equ REGBASE+$244 ;portt pull device enable

PPST           equ REGBASE+$245 ;portt pull polarity select

PTS            equ REGBASE+$248 ;ports data register

PTIS           equ REGBASE+$249 ;ports input register

DDRS           equ REGBASE+$24a ;ports direction register

RDRS           equ REGBASE+$24b ;ports reduced drive register

PERS           equ REGBASE+$24c ;ports pull device enable

PPSS           equ REGBASE+$24d ;ports pull polarity select

WOMS           equ REGBASE+$24e ;ports wired or mode register

PTM            equ REGBASE+$250 ;portm data register

PTIM           equ REGBASE+$251 ;portm input register

DDRM           equ REGBASE+$252 ;portm direction register

RDRM           equ REGBASE+$253 ;portm reduced drive register

PERM           equ REGBASE+$254 ;portm pull device enable

PPSM           equ REGBASE+$255 ;portm pull polarity select

WOMM           equ REGBASE+$256 ;portm wired or mode register

MODRR          equ REGBASE+$257 ;portm module routing register

PTP            equ REGBASE+$258 ;portp data register

PTIP           equ REGBASE+$259 ;portp input register

DDRP           equ REGBASE+$25a ;portp direction register

RDRP           equ REGBASE+$25b ;portp reduced drive register

PERP           equ REGBASE+$25c ;portp pull device enable

PPSP           equ REGBASE+$25d ;portp pull polarity select

PIEP           equ REGBASE+$25e ;portp interrupt enable register

PIFP           equ REGBASE+$25f ;portp interrupt flag register

PTH            equ REGBASE+$260 ;porth data register

PTIH           equ REGBASE+$261 ;porth input register

DDRH           equ REGBASE+$262 ;porth direction register

RDRH           equ REGBASE+$263 ;porth reduced drive register

PERH           equ REGBASE+$264 ;porth pull device enable

PPSH           equ REGBASE+$265 ;porth pull polarity select

PIEH           equ REGBASE+$266 ;porth interrupt enable register

PIFH           equ REGBASE+$267 ;porth interrupt flag register

PTJ            equ REGBASE+$268 ;portj data register

PTIJ           equ REGBASE+$269 ;portj input register

DDRJ           equ REGBASE+$26a ;portj direction register

RDRJ           equ REGBASE+$26b ;portj reduced drive register

PERJ           equ REGBASE+$26c ;portj pull device enable

PPSJ           equ REGBASE+$26d ;portj pull polarity select

PIEJ           equ REGBASE+$26e ;portj interrupt enable register

PIFJ           equ REGBASE+$26f ;portj interrupt flag register

CAN4CTL0       equ REGBASE+$280 ;can4 control register 0

CAN4CTL1       equ REGBASE+$281 ;can4 control register 1

CAN4BTR0       equ REGBASE+$282 ;can4 bus timing register 0

CAN4BTR1       equ REGBASE+$283 ;can4 bus timing register 1

CAN4RFLG       equ REGBASE+$284 ;can4 receiver flags

CAN4RIER       equ REGBASE+$285 ;can4 receiver interrupt enables

CAN4TFLG       equ REGBASE+$286 ;can4 transmit flags

CAN4TIER       equ REGBASE+$287 ;can4 transmit interrupt enables

CAN4TARQ       equ REGBASE+$288 ;can4 transmit message abort control

CAN4TAAK       equ REGBASE+$289 ;can4 transmit message abort status

CAN4TBSEL      equ REGBASE+$28a ;can4 transmit buffer select

CAN4IDAC       equ REGBASE+$28b ;can4 identfier acceptance control

CAN4RXERR      equ REGBASE+$28e ;can4 receive error counter

CAN4TXERR      equ REGBASE+$28f ;can4 transmit error counter

CAN4IDAR0      equ REGBASE+$290 ;can4 identifier acceptance register 0

CAN4IDAR1      equ REGBASE+$291 ;can4 identifier acceptance register 1

CAN4IDAR2      equ REGBASE+$292 ;can4 identifier acceptance register 2

CAN4IDAR3      equ REGBASE+$293 ;can4 identifier acceptance register 3

CAN4IDMR0      equ REGBASE+$294 ;can4 identifier mask register 0

CAN4IDMR1      equ REGBASE+$295 ;can4 identifier mask register 1

CAN4IDMR2      equ REGBASE+$296 ;can4 identifier mask register 2

CAN4IDMR3      equ REGBASE+$297 ;can4 identifier mask register 3

CAN4IDAR4      equ REGBASE+$298 ;can4 identifier acceptance register 4

CAN4IDAR5      equ REGBASE+$299 ;can4 identifier acceptance register 5

CAN4IDAR6      equ REGBASE+$29a ;can4 identifier acceptance register 6

CAN4IDAR7      equ REGBASE+$29b ;can4 identifier acceptance register 7

CAN4IDMR4      equ REGBASE+$29c ;can4 identifier mask register 4

CAN4IDMR5      equ REGBASE+$29d ;can4 identifier mask register 5

CAN4IDMR6      equ REGBASE+$29e ;can4 identifier mask register 6

CAN4IDMR7      equ REGBASE+$29f ;can4 identifier mask register 7

CAN4RXFG       equ REGBASE+$2a0 ;can4 rx foreground buffer thru +$2af

CAN4TXFG       equ REGBASE+$2b0 ;can4 tx foreground buffer thru +$2bf

* end registers

;vector_table.inc

; ----------------------------------------------------

; This vector table is intended for use with code

; downloaded into Flash using the resident bootloader

; in the Dragon12 board.

; ------------------ VECTOR TABLE --------------------

org        $EF8C

fdb        Handler_0		;$EF8C: PWM emergency shutdown

fdb        Handler_0		;$EF8E: PortP

fdb        Handler_0		;$EF90: CAN4TX

fdb        Handler_0		;$EF92: CAN4RX

fdb        Handler_0		;$EF94: CAN4ERR

fdb        Handler_0		;$EF96: CAN4WU

fdb        Handler_0		;$EF98: CAN3TX

fdb        Handler_0		;$EF9A: CAN3RX

fdb        Handler_0		;$EF9C: CAN3ERR

fdb        Handler_0		;$EF9E: CAN3WU

fdb        Handler_0		;$EFA0: CAN2TX

fdb        Handler_0		;$EFA2: CAN2RX

fdb        Handler_0		;$EFA4: CAN2ERR

fdb        Handler_0		;$EFA6: CAN2WU

fdb        Handler_0		;$EFA8: CAN1TX

fdb        Handler_0		;$EFAA: CAN1RX

fdb        Handler_0		;$EFAC: CAN1ERR

fdb        Handler_0		;$EFAE: CAN1WU

fdb        Handler_0		;$EFB0: CAN0TX

fdb        Handler_0		;$EFB2: CAN0RX

fdb        Handler_0		;$EFB4: CAN0ERR

fdb        Handler_0		;$EFB6: CAN0WU

fdb        Handler_0		;$EFB8: FLASH

fdb        Handler_0		;$EFBA: EEPROM

fdb        Handler_0		;$EFBC: SPI2

fdb        Handler_0		;$EFBE: SPI1

fdb        Handler_0		;$EFC0: IIC Bus

fdb        Handler_0		;$EFC2: BDLC

fdb        Handler_0		;$EFC4: CRG self-clock-mode

fdb        Handler_0		;$EFC6: CRG PLL Lock

fdb        Handler_0		;$EFC8: Pulse Accumulator B Overflow

fdb        Handler_0		;$EFCA: Modulus Down Counter underflow

fdb        Handler_0		;$EFCC: PORTH

fdb        Handler_0		;$EFCE: PORTJ

fdb        Handler_0		;$EFD0: ATD1

fdb        Handler_0		;$EFD2: ATD0

fdb        Handler_0		;$EFD4: SCI1 Serial System

fdb        Handler_0		;$EFD6: SCI0 Serial System

fdb        Handler_0		;$EFD8: SPI0

fdb        Handler_0		;$EFDA: Pulse Accumulator Input Edge

fdb        Handler_0		;$EFDC: Pulse Accumulator A Overflow

fdb        Handler_0		;$EFDE: Enhanced Capture Timer Overflow

fdb        Handler_0		;$EFE0: Enhanced Capture Timer Channel 7

fdb        Handler_0		;$EFE2: Enhanced Capture Timer Channel 6

fdb        Handler_0		;$EFE4: Enhanced Capture Timer Channel 5

fdb        Handler_0		;$EFE6: Enhanced Capture Timer Channel 4

fdb        Handler_0		;$EFE8: Enhanced Capture Timer Channel 3

fdb        Handler_0		;$EFEA: Enhanced Capture Timer Channel 2

fdb        Handler_0		;$EFEC: Enhanced Capture Timer Channel 1

fdb        Handler_0		;$EFEE: Enhanced Capture Timer Channel 0

fdb        RTI_ISR		;$EFF0: Real Time Interrupt (RTI)

fdb        Handler_0		;$EFF2: IRQ

fdb        Handler_0		;$EFF4: XIRQ

fdb        Handler_0		;$EFF6: Software Interrupt (SWI)

fdb        Handler_0		;$EFF8: Illegal Instruction Trap

fdb        start		;$EFFA: COP Failure Reset

fdb        Handler_0		;$EFFC: Clock Monitor Fail Reset

fdb        start		;$EFFE: /RESET

Data Sheets: